WS 2025/26

Lab course: Human Robot Interaction

FINAL COURSE REPORT

Chair of Autonomous Systems and Mechatronics

Friedrich-Alexander-Universität Erlangen-Nürnberg

TEAM 1 :

Velayudhan Aravindakshan - ju08rufe

Akhilan David Karthikeyan - ug39owys

Wei Zhang - jo07wuwe

Task 1

- Trajectory planning considerations

When planning a trajectory in a joint space, major steps to be considered for successful execution are:

- Joint limits: The planned joint positions must remain within the physical limits of each joint’s rotation/translation to avoid mechanical failure or errors.

- Velocity or acceleration limits: The rate of change of joint positions and velocity must be limited to values that the robot’s motor and controllers can physically achieve. Exceeding these limits will result in an error or failed execution.

- Rate of execution: In the rospy.rate() within the publishing node, the rate has to be fast enough for smooth motion but not so fast that the controller is overwhelmed or overloaded.

- Collision Avoidance: The intermediate waypoints along the straight line in joint space must not lead the robot’s physical links into collision with itself, the environment, or other objects.

b.	End Effector behaviour in linearly planned joint space

When traversing a trajectory that was planned linearly in joint space, the behaviour of the end effector in Cartesian space will typically be non-linear and curved. This is because the kinematic equations that map joint angles to the end effector’s Cartesian position are complex trigonometric functions. A linear path in the input space does not generally result in a straight line in the output space. The end effector’s path will follow an arc determined by the combined motion of all seven joints.

c.	Other  Levels of Robot Commands

Control was applied at the position level in Exercise, where the desired joint positions were commanded. Another level of robot commands is the velocity level (or rate level)

- Position Control: Commands the robot to move to a specific absolute target position. The resulting motion is highly dependent on the initial starting position.

- Velocity Control: Commands the robot to move at a specific rate of change. The robot executes the commanded motion regardless of its current position until the command is stopped or a limit is reached.

d. 	Reaching a certain point or moving linearly in the workspace

The end effector is used to move along a straight line in the workspace or to reach a specific Cartesian pose; in these cases, Inverse Kinematics is primarily employed.

- Inverse Kinematics: The goal Cartesian pose is inputted to an inverse kinematics solver, which calculates the required set of joint angles that will place the end effector at the exact pose.

- Cartesian Trajectory Planning: For linear movement, the desired straight line path is first defined as a series of intermediate Cartesian waypoints. The inverse kinematics solver is then used at each intermediate point to find the corresponding joint angle configuration.

- Joint Trajectory Execution: The calculated joint angle waypoints are executed by the robot’s position controller, while ensuring the intermediate points realise a straight line in the Cartesian space.

e. 	Calculating End Effector Orientation from the Rotation Matrix

The rotation matrix describes the orientation of the end effector frame relative to the base frame. A common method for calculating the orientation using three angles from a rotation matrix R is through Euler Angles or Roll-Pitch-Yaw angles.

The general procedure is:

- Define the Angle sequence (Euler Angles): An orientation is represented by three successive rotations about defined axes(eg, Z-Y-X, Z-Y-Z). A common sequence in robotics is Z-Y-X(RPY), which represents rotation about the base Z-axis(Yaw), followed by rotation about the new Y-axis(Pitch), and finally a rotation about the newest X-axis(Roll).

- Equate Matrices: The general rotation matrix R(ɸ,θ,φ) for the chosen sequence(eg, Z-Y-X) is mathematically expanded in terms of the three angles(ɸ,θ,φ). The expanded matrix is then equated to the calculated

- Solve for Angles: By analysing the matrix components, trigonometric equations are derived to solve for the three angles.

A more robust alternative to RPY angles for representing orientation is using a Quaternion, which uses four parameters and does not suffer from singularities.

Task 2

- Problems with Inverse Kinematics for Trajectories

When a Cartesian trajectory is converted to a joint space trajectory using Inverse Kinematics, the primary problem is the continuity of the solutions.

Inverse Non-uniqueness and discontinuity: Inverse kinematics solutions for a redundant robot like the Franka Emika that we are using for the experiment are not unique, i.e there are multiple joint configurations for the same end effector pose. The inverse kinematics solver can switch between these multiple solutions between consecutive steps, even if the Cartesian path is perfectly smooth. This jump creates discontinuities in the joint trajectory, resulting in sudden, large changes in joint angles and causing jerky, high-velocity motion for the robot.

If we differentiate the methods

| Method | Advantages | Disadvantages |
| --- | --- | --- |
| Cartesian Space Planning with the use of the Inverse Kinematics solver | This results in a predictable, straight-line path in the physical workspace | Requires complex Inverse kinematics calculations at every step. It is affected by singularities and discontinuities in the joint space. |
| Joint Space Planning | Computationally simpler, i.e there is no need for an Inverse Kinematics solver. It is guaranteed to have continuous joint motion. | Results in a non-linear, curved path in the Cartesian space, making the end effector’s trajectory difficult to predict. |

- Event-Based Message Transmission in ROS

For events that occur at certain, distinct moments, the most suitable type of ROS transmission would be a Service or an Action.

- ROS Services: Are a request/reply mechanism. The monitoring node would act as a client, sending a request to the motion node, which would execute the action once and reply when complete. This is ideal for single, synchronous events.

- ROS Actions: Are typically used for long-running, preemptable tasks. They allow for sending a goal and receiving continuous feedback on the status, which is better than a service for a persistent change, like a new motion vector.

- Problems with Deriving External Forces from Joint Moments

When deriving external forces from measured joint moments, problems can arise because the joint moment measurement contains the contributions from:

- Gravity: The static load of the links - g(q)

- Inertia: The robot’s own acceleration/deceleration - M(q)q’’ + C(q,q’)q’

- Friction: Unmodeled friction in the joints

To accurately isolate the external force, the dynamic model of the robot must be precisely known, and its contributions must be subtracted:

Any modelling error(eg, in mass or centre of gravity) directly translates into an error in the estimated external force.

| Sensor Type | Advantages | Disadvantages |
| --- | --- | --- |
| End Effector Force Sensor | Provides a direct, precise measurement of torque/force at the contact point. It can measure forces in all directions(up to six axes) | Adds weight and bulk to the end effector, reducing payload capacity. Only measures forces at the tip; forces on other links are not detected directly |
| Joint Torque Sensor | Provides force information across the entire robot structure. Cheaper/lighter than a 6-axis end effector sensor. | Requires accurate dynamic modelling to compute external forces |

Unmeasurable Forces: Yes, there are configurations where certain external forces cannot be measured with joint sensors only. If an external force is applied at a link but perfectly aligned with the joint axis ( a configuration where the force does not create a moment/torque about that joint axis), the joint sensor will not register the presence of that force.

- Interaction with Links under Task Space Impedance Control

The task space impedance controller used on the Franka Emika robot is designed to create a virtual spring-damper behaviour fixed at the end effector (in task space).

When an external force occurs on the links and not the end effector, the reaction changes:

- Intended Compliant Behaviour: The robot will still react compliantly because the external force creates joint moments. These moments are internally measured, and the impedance control law calculates the corresponding motor torques to produce a reaction consistent with the virtual spring-damper at the end effector.

- Unintended End Effector Motion: The resulting end effector motion and the corresponding joint positions q are calculated to satisfy the impedance relationship at the end effector. The compliance is experienced globally, but the local yielding at the point of contact on the link may not be perfectly aligned with the desired task space compliance, as the control loop is optimising the end effector’s response, not the link’s response.

Task 3

- Review of Pick and Place Experiments

The Pick-and-Place experiments were performed by 10 non-skilled volunteers to analyse user behaviour during a representative task, comparing the Single Point (SP) method with the Extended Patch (EP) method.

The paper’s findings consistently show that the Extended Patch(EP) method is superior to the Single Point(SP) method for cooperative grasping tasks involving translations and rotations:

Completion Time: The EP method significantly decreased the average completion time by approximately 16% (39.78s for EP vs. 47.41s for SP) during the test phase.

Contact Permancence/precision (Rotational Stability): This was the most relevant result related to contact stability. With the SP method, the box and plate ended with a large average angle of 30.1° about the plate y-axis, indicating a significant rotational detachment. With the EP method, the rotational detachment was much smaller (average 4.3 degrees about the y-axis and 11.4 degrees about the z-axis).

Relationship to Lab Experience:

The lab experience mirrored the finding that the SP method struggles with rotational stability and contact. We performed the cooperative lifting by implementing Algorithm 1 based on the Single Point method. We also observed that even a slight misalignment by us for the cooperative lifting causes the basic SP model to fail at maintaining stable contact, resulting in the robot being unstable or losing contact easily. We also observed that there was constant jerky behaviour of the robot that decreased performance and made us to to put forth more effort for the cooperative lifting task.

- A robot’s low level control modality defines the quantity it directly commands to its joint actuators.

| Task/Level | Modality/Quantity Commanded | Tasks Used in |
| --- | --- | --- |
| Position Level | Joint Angles | Task 1 & 2 (Joint Trajectory, Cartesian Trajectory via Inverse Kinematics) |
| Velocity Level | Joint or Cartesian Velocity(q’,p’) | Task 3 (Cooperative Lifting, using a Cartesian Velocity Controller) |
| Torque Level | Joint Torques | Task 2 (Impedance Control internally computes and commands torques) |

Other Possibilities/Levels:

- Acceleration Control: Commands the joint accelerations (q’’), often used in complex, model-based control schemes.

- Admittance Control (External Force Input): While typically outputting position/velocity commands to a subordinate position controller, its function is often categorised as a compliant control modality, distinct from pure position control.

| Feature | ROS Services | ROS Actions |
| --- | --- | --- |
| Communication Pattern | Synchronous request/reply | Asynchronous client/server, with goal/feedback/result |
| Task Duration | Short, non-preemptable processes | Long-running tasks (e.g., navigation, complex manipulation) |
| Feedback | No feedback until the response is returned | Provides periodic feedback (status updates) to the client |
| Preemption | Cannot be cancelled once started | Can be cancelled (preempted) by the client before completion |

| Type | Advantages | Disadvantages |
| --- | --- | --- |
| ROS Services | Simpler implementation. Guarantees a response. Good for simple, instantaneous events or queries that run very quickly. | Blocks the client until completion. Not suitable for long-running tasks. Provides no intermediate feedback. |
| ROS Actions | Ideal for long-running tasks. Provides real-time feedback on progress. Can be preempted (cancelled or modified) while running. | More complex implementation than services. Higher computational overhead. |

In our task 3, ROS Services were sufficient because they were used to trigger a single state change, which is an instantaneous event requiring confirmation, not continuous feedback.

Task 4

Teleoperation Pipeline Flowchart

For the teleoperation pipeline visualisation, we have utilised the rqt_graph command. This pictorial representation provides a clearer explanation of the node and its associated topics.

In this graph:

- Elliptical shapes represent nodes (executable processes, such as scripts or controller programs).

- Rectangular shapes represent Topics (the communication channels).

- Arrows indicate the direction of data flow (from Publisher -> Topic -> Subscriber).

The pipeline can be divided into three main functional segments: Sensor Feedback, Teleoperation Logic, and Robot Control.

### a. Sensor Feedback Pipeline

This chain processes the raw force reading from the sensor to make it usable for haptic feedback.

- Raw Data Acquisition: The /serial/_node (Node, representing the Arduino microcontroller) publishes the raw force data to the /pub/_force (Topic).

- Data Processing: The /mapping/_node (Node) subscribes to /pub/_force and performs the normalisation (0-1023 range mapped to 0-1). It then publishes the processed, mapped force value to the /force/_mapped (Topic).

### b. Teleoperation and Gripper Logic

The core logic node gathers all inputs (gamepad and force sensor) and generates all high-level commands.

- Command Input & Feedback: The /teleoperation (Node) subscribes to /force/_mapped to receive the 0-1 float value. This is used to drive the vibration feedback on the gamepad. Simultaneously, this node reads the gamepad input (Analog sticks and buttons) and calculates two sets of commands:

  - It publishes the desired Cartesian Velocity commands to the /cartesian/_velocity (Topic).

  - It publishes the desired Gripper Width to the /gripper/_width (Topic).

### c. Robot and Gripper Control

This segment takes the high-level commands and passes them to the robot's low-level controllers.

- Arm Motion Control: The /franka/_control (Node) represents the main robot controller launched for Cartesian velocity. It subscribes to /cartesian/_velocity to execute the robot arm motion.

- Gripper Control: The /gripper/_manager (Node) subscribes to /gripper/_width and translates the human's desired width command into the specific message format required by the Franka gripper.

  - The /gripper/_manager publishes these commands to the /franka/_gripper (Node).

  - The /franka/_gripper Node (or the topics surrounding it, like /franka/_gripper/move, /franka/_gripper/grasp, etc.) manages the actual hardware control of the gripper fingers.

d. Robot State Flow (Status Reporting)

The bottom right section displays the flow of sensor and state data from the robot, which is commonly used by other nodes for awareness and debugging purposes.

- The /franka/_state/_controller (Node) publishes continuous sensor and state information (like joint torques, end effector pose, etc.) to the /joint/_states (Topic) and /franka/_state/_controller/joint/_states (Topic).

- These state topics are then consumed by other components like /joint/_state/_publisher and /robot/_state/_publisher (Nodes), which are essential for visualising the robot accurately in tools like RViz.

Task 5

### a)Comparison of HRI Theory and Practical Experience

Based on the implementation of QiChat and graphical programming, several alignments and discrepancies with the textbook chapters were observed:

- Verbal Syntax and Structure: The use of user rules (u:) and subrules (u1:, u2:) in QiChat directly aligns with the theory of hierarchical dialogue management in Chapter 7 part 2. The implementation showed that while a robot can follow linear paths, human-robot interaction (HRI) often requires "staying in scope" to maintain context.

- Speech Recognition Constraints: Chapter 7 part 3, discusses the limitations of speech perception. The Confidence threshold settings practically evidenced this. In the lab, recognition was not perfect; environmental noise or accents often required adjusting the threshold or using Concepts to provide the robot with a list of synonyms to increase the "hit rate.

- Simulation vs. Reality: A significant departure from theoretical "ideal" interaction was the difference between the simulated and real NAO. While the code logic worked in simulation, real-world factors such as microphone sensitivity and the simulator's inability to display certain LED or motor behaviours highlighted the "embodiment" challenges discussed in Chapter 6, Part 2.

### b) Comparison of Purely Verbal vs. Multimodal Interaction

The following insights were derived from the communication.

- The "Liveliness" Factor: Interaction without non-verbal elements felt mechanical and stagnant. Disabling Autonomous Life removed the small, natural movements that create the impression of a living entity. Without these, it was harder to tell if the robot was "listening" or had crashed during pauses in the dialogue.

- Gestures and Timing: Adding Annotated Text or Tags in a specific way significantly improved the user experience. For instance, using a "hello" animation tag during a greeting made the interaction feel more intentional and social.

Task 6

Through multiple tests done in the lab, the following observations were obtained. Each object was tested multiple times, and the average of 5 observations is taken into consideration.

| Object | Method | Correctly Detected | Incorrectly Detected | Not Detected |
| --- | --- | --- | --- | --- |
| Soccerball | NAO Integrated | 3 | 1 | 1 |
|  | YOLOv5 | 4 | 0 | 1 |
| Mug | NAO Integrated | 1 | 2 | 2 |
|  | YOLOv5 | 4 | 1 | 0 |
| Bottle | NAO Integrated | 2 | 1 | 2 |
|  | YOLOv5 | 4 | 0 | 1 |
| Scissors | NAO Integrated | 1 | 0 | 4 |
|  | YOLOv5 | 5 | 0 | 0 |

- Comparison of Result

Accuracy: The YOLOv5 neural network performs significantly better than the NAO’s integrated function. While the NAO often struggles with varying backgrounds and lighting conditions, the CNN is more robust due to its ability to encode image-specific features and utilise data augmentation.

Learning Process: The NAO method requires manual "learning" via Choregraphe by drawing boundaries around a physical object in real-time. In contrast, the YOLO method uses a large dataset (like COCO) and transfer learning, allowing it to recognise broad categories of objects without needing to see the specific item beforehand.

Operational Constraints: The NAO’s integrated recognition is limited by its “on-board” database and is highly sensitive to the environment. YOLO’s performance is superior, in this lab setup, is limited by the acquisition delay caused by the Wifi connection when transferring images from the robot to the PC processing.

- NAO’s Object Detection Algorithm

Based on the online documentation, NAO uses a Sift-based (Scale-Invariant Feature Transform) algorithm for its “Vision Recognition”. The SIFT-based approach used by NAO relies on matching specific visual "keypoints". This is why it works best on objects with high contrast and many details (like the soccer ball) and fails on plain or reflective objects (like some mugs or scissors). The YOLOv5 (CNN) approach is the current state-of-the-art because it evaluates the entire image context rather than just isolated keypoints, leading to higher success rates.